IAI ELECYLINDER

Sample Project File

For QVGA

Technical Guide　　　　　　Ver1.30

Revision History

| No. | Date | Details |
| --- | --- | --- |
| 01 | Oct. 15, 2019 | New |
| 02 | Jul. 20, 2020 | Added models that can be connected. |
| 03 | Sep. 14, 2020 | Elecylinder type added. |
| 04 | JuI. 1, 2021 | Elecylinder type added. |
| 05 | Nov. 10, 2022 | Elecylinder type added. New model addition STM6000. Compatible model series name changed LT-4000M -> LT4000. |

目次

1. Overview	1

2. Notes	1

3. Limitations	1

4. How to use this project file	2

5. Device Configuration	4

5.1. System Configuration	4

5.2. Target model list of the display unit	5

5.3. Software	9

5.4. Connected device	9

5.5. How to connect and communication cable diagram	11

5.5.1. System Configuration	11

5.5.2. Cable Diagram	13

5.6. Communciation Settings	34

5.6.1. GP-Pro EX Communication Settings	34

5.6.2. Communication Settings of Devices/PLCs	35

5.6.3. Indirect Device Settings	35

5.6.4. Device-Specific Settings	36

6. Screen Configuration	37

6.1. Screen Type	37

6.2. Screen Transition	39

7. Details of Screens	40

7.1. Home Screen (B0001: Edit allowed）	40

7.1.1. Overview	40

7.1.2. Screen Image	40

7.2. Axis Select (B9990: Edit not allowed)	41

7.2.1. Overview	41

7.2.2. Screen Image	41

7.3. Axis No./Name (B9992: Edit not allowed)	42

7.3.1. Overview	42

7.3.2. Screen Image	42

7.4. Forward Condition Screen (B9993: Edit not allowed)	43

7.4.1. Overview	43

7.4.2. Screen Image	43

7.5. Backward Condition Screen (B9994: Edit not allowed)	45

7.5.1. Overview	45

7.5.2. Screen Image	45

7.6. Position Setting Screen (B9995: Edit not allowed)	47

7.6.1. Overview	47

7.6.2. Screen Image	47

7.7. Simple Data Setting (B9996: Edit not allowed)	51

7.7.1. Overview	51

7.7.2. Screen Image	51

7.8. Transfer Confirmation Screen (W2000: Edit not allowed）	53

7.8.1. Overview	53

7.8.2. Screen Image	53

8. Global D Script	54

9. Address Map	54

9.1. List of used internal addresses	54

9.2. Variable List	54

10. Text Table	54

10.1. Table1（Table 1）	54

11. Incorporating project data	55

11.1. To create a project data file based on the file	55

11.2. When incorporating the file in the created project file	55

11.2.1. Device/PLC Settings	56

11.2.2. Import and export of text table	57

11.2.3. Copy From Another Project	58

12. Incorporating project data	60

12.1. Change the communication port.	60

12.2. Communication Setting Change (the communication type included)	61

# Overview

This sample project file is for connecting IAI ELECYLINDER to GP4000 / SP5000 / LT4000 / ST6000

/ STM6000 series. The following functions are provided for more convenient use of the ELECYLINDER.

・Checking setting status of up to 16 ELECYLINDERs

・Easily configuring forward and backward settings of the ELECYLINDER and the position settings

・Checking setting status of every ELECYLINDER one by one when checking settings

・Configuring settings of axis numers and names

・Manual mode – JOG operation and Inching -

・Homing is possible when it is not completed

# Notes

・The intellectual property rights for the files provided by Schneider Electric Japan Holdings Ltd. belong to us

・Downloaded files and the data extracted from those files are not guarantees of our product specifications. Please be aware of this fact.

・Please use this service at your own risk.

・In any case, this not intended as a warranty for any work for a system that makes use of the data on these screens.

・For the models that can run on this program, please refer to the chapter of [5.2. Target model list of the display unit] in this sample project file.

・Any modifications made to this service by a customer are entirely at the responsibility of the customer.

・Please be aware that we cannot respond to any inquiries for the purpose of modifying these data.

・The content and information in the data on these screens and documentation are subject to change without prior notification.

# Limitations

Typcal features of GP4000 / SP5000 / LT4000 /ST6000 / STM6000 series are used for this screen data.

When using the sample project file, be sure to refer to our product manual or the connection device manual, including the usage restrictions and safety precautions. In addition, please be aware that we are unable to accept responsibility for damage arising from use of this sample project file -modification and diversion included-, loss of customer opportunity or profit arising from the malfunction of our product, damage arising from special circumstances whether or not we had foreknowledge of those circumstances, secondary damage, compensation for accidents, damage to our products, or other business-related guarantees.

# How to use this project file

When using this project file (hereinafter referred to as “the file”), be sure to confirm the following details:

- Confirm the connection configuration and the communication settings.

Referring to [5. Device Configuration], confirm the connection configuration to be used and the communication settings of the ELECYLINDER unit. The initial settings configured in the file are as described in [Table 4-1 Connection Configuration Initial Settings]. When the connection configuration to be used is the same as the initial settings in the file, transfer the data to the programmable display unit (hereinafter referred to as “the display unit”) using the transfer feature of GP-Pro EX. For use in another configuration, change the settings in the file before transferring them.

Table 4-1 Connection Configuration Initial Settings

| Item | Default | Details |
| --- | --- | --- |
| Display Unit Type | For the file for QVGA, Model：GP-4301T | 5.2. Target model list of the display unit Change it to the display unit type you use before use. |
|  | For the file for VGA, Model：GP-4501T(Analog Touch Panel) |  |
|  | For the file for WVGA, Box Type： SP-5B10 Display Type： SP-5400WA (WVGA (800*480)) |  |
| System Configuration | When a SIO converter is not used, | 5.5.1. System Configuration 5.5.2. Cable Diagram 5.6.1. GP-Pro EX Communication Settings The cable diagram and the communication method differ depending on with or without use of a SIO converter, a connection port, and a cable type to be used. When different from the initial settings, check every item again and change them according to the connection configuration you will use. |
| Cable Diagram | Cable Diagram 2 |  |
| Display Unit Connection Port | COM2 |  |
| Communication method | RS-422/485(2-wire type) |  |
| Baud rate | 38400 (bps) | 5.6.1. GP-Pro EX Communication Settings The communication settings must be the same as those of the ELECYLINDER to be connected. |
| Data length | 8 bits |  |
| Parity | None |  |
| Stop Bit | 1 bit |  |

- How to combine with other files
In GP-Pro EX, select [Project] → [Utilities] → [Copy from Another Project].

For further details, refer to “from Startup to Shutdown” in our reference manual.

However, there are issues to be aware of, such as overlapping screen numbers, so also refer to sections 3) and later.

[Copy from Another Project] is for screen data only. If the following settings are not configured except screen data,

・Communication Settings [5.6. Communication Settings]

・Text Table [10. Text Table]

the operation will be affected.

For details, refer to 11. Incorporating project data in the file.

- Screen numbers when combining
Don’t change the screen numbers in the file. Note that the operation will be affected if you change    them. At the time of combining with other project files, when there are duplicate screen numbers, the screens are overwritten. For the screen numbers used in the file, refer to [6.2. Screen Transition].

When combining with (2), it is possible to designate a copy destination screen number before starting to copy. Before combining, be sure to either designate a screen number when copying, or change the screen number in advance.

- Changing addresses
Don’t change the addresses of the connected devices used in the file. If you change them, the operation will be affected. Also, do not use the internal addresses (USER address) and the connection device addresses described in [9.1. Address Map].

# Device Configuration

## System Configuration

GP4000 series

LT4000 series

SIO converter

Portrait: RCB-TU-SIO-A

Landscape: RCB-TU-SIO-B

RS232C cable

or

RS-485 cable

RS-485 cable

ELECYLINDER

Max. of connection units: 16 units

Figure 5-1  System Configuration

* The power supply specifications of GP4000 and LT4000 series differ depending on a model.

* For this file, up to 16 ELECYLINDERs can be connected per display unit.

* For details of connection, refer to [5.5．How to connect and communication cable diagram].

* When there are duplicate unit numbers, it’s hard to tell one from another. Be sure to avoid duplicate unit numbers when configuring settings.

## Target model list of the display unit

GP4000 series and LT4000 series can use the file here.

For details of supported target models, see the following table.

The model names that can be selected on GP-Pro EX are stated.

Table 5-1  Supported target models

| Series | Model | Target model | Note |
| --- | --- | --- | --- |
| GP4000 Series | GP-4104 |  |  |
|  | GP-4105 |  |  |
|  | GP-4106 |  |  |
|  | GP-4107 |  |  |
|  | GP-4114T |  |  |
|  | GP-4115T |  |  |
|  | GP-4116T |  |  |
|  | GP-4115T3 |  |  |
|  | GP-4201T | √ |  |
|  | GP-4201TM (Modular Type) | √ |  |
|  | GP-4201TW | √ |  |
|  | GP-4203T | √ |  |
|  | GP-4301T | √ | *1 |
|  | GP-4301TM (Modular Type) | √ |  |
|  | GP-4301TW | √ |  |
|  | GP-4303T | √ |  |
|  | GP-4311HT | √ |  |
|  | GP-4401T |  |  |
|  | GP-4401WW |  |  |
|  | GP-4501T (Analog Touch Panel) |  |  |
|  | GP-4501T (Matrix Touch Panel) |  |  |
|  | GP-4501TW |  |  |
|  | GP-4503T |  |  |
|  | GP-4521T |  |  |
|  | GP-4601T (Analog Touch Panel) |  |  |
|  | GP-4601T (Matrix Touch Panel) |  |  |
|  | GP-4603T |  |  |
|  | GP-4621T |  |  |
|  | GP-4G01 VGA (640*480) |  |  |
|  | GP-4G01 SVGA (800*600) |  |  |
|  | GP-4G01 WVGA (800*480) |  |  |
|  | GP-4000M (Rear Modular Type) | √ |  |
| LT4000 Series | LT-4201TM (Modular Type DIO) | √ |  |
|  | LT-4201TM (Modular Type Analog) | √ |  |
|  | LT-4301TM (Modular Type DIO) | √ |  |
|  | LT-4301TM (Modular Type Analog) | √ |  |
|  | LT-4000M (Rear Module DIO) | √ |  |
|  | LT-4000M (Rear Module Analog) | √ |  |
| SP5000 Series (SP-5B00) | SP-5400WA WVGA (800*480) |  |  |
|  | SP-5500TP VGA (640*480) |  |  |
|  | SP-5500TP SVGA (800*600) |  |  |
|  | SP-5500WA WXGA (1280*800) |  |  |
|  | SP-5600TA XGA (1024*768) |  |  |
|  | SP-5600TP VGA (640*480) |  |  |
|  | SP-5600TP SVGA (800*600) |  |  |
|  | SP-5600TP XGA (1024*768) |  |  |
|  | SP-5600WA WXGA (1280*800) |  |  |
|  | SP-5660TP VGA (640*480) |  |  |
|  | SP-5660TP SVGA (800*600) |  |  |
|  | SP-5660TP XGA (1024*768) |  |  |
|  | SP-5700TP VGA (640*480) |  |  |
|  | SP-5700TP SVGA (800*600) |  |  |
|  | SP-5700TP XGA (1024*768) |  |  |
|  | SP-5700WC FWXGA (1366*768) |  |  |
|  | SP-5800WC FWXGA (1366*768) |  |  |
|  | DC Power Supply Adapter SVGA (800*600) |  |  |
|  | DC Power Supply Adapter XGA (1024*768) |  |  |
| SP5000 Series (SP-5B10) | SP-5400WA WVGA (800*480) |  |  |
|  | SP-5500TP VGA (640*480) |  |  |
|  | SP-5500TP SVGA (800*600) |  |  |
|  | SP-5500WA WXGA (1280*800) |  |  |
|  | SP-5600TA XGA (1024*768) |  |  |
|  | SP-5600TP VGA (640*480) |  |  |
|  | SP-5600TP SVGA (800*600) |  |  |
|  | SP-5600TP XGA (1024*768) |  |  |
|  | SP-5600WA WXGA (1280*800) |  |  |
|  | SP-5660TP VGA (640*480) |  |  |
|  | SP-5660TP SVGA (800*600) |  |  |
|  | SP-5660TP XGA (1024*768) |  |  |
|  | SP-5700TP VGA (640*480) |  |  |
|  | SP-5700TP SVGA (800*600) |  |  |
|  | SP-5700TP XGA (1024*768) |  |  |
|  | SP-5700WC FWXGA (1366*768) |  |  |
|  | SP-5800WC FWXGA (1366*768) |  |  |
|  | DC Power Supply Adapter SVGA (800*600) |  |  |
|  | DC Power Supply Adapter XGA (1024*768) |  |  |
| SP5000 Series (SP-5B40, SP-5B41, SP-5B41*) | SP-5400WA WVGA (800*480) |  |  |
|  | SP-5500TP SVGA (800*600) |  |  |
|  | SP-5500WA WXGA (1280*800) |  |  |
|  | SP-5600TA XGA (1024*768) |  | SP-5B41, SP-5B41* only |
|  | SP-5600TP SVGA (800*600) |  |  |
|  | SP-5600TP XGA (1024*768) |  |  |
|  | SP-5600WA WXGA (1280*800) |  |  |
|  | SP-5660TP SVGA (800*600) |  |  |
|  | SP-5660TP XGA (1024*768) |  |  |
|  | SP-5700TP SVGA (800*600) |  |  |
|  | SP-5700TP XGA (1024*768) |  |  |
|  | SP-5700WC FWXGA (1366*768) |  | SP-5B41, SP-5B41* only |
|  | SP-5800WC FWXGA (1366*768) |  | SP-5B41, SP-5B41* only |
|  | DC Power Supply Adapter SVGA (800*600) |  |  |
|  | DC Power Supply Adapter XGA (1024*768) |  |  |
|  | DC Power Supply Adapter HD720p (1280*720) |  | SP-5B41, SP-5B41* only |
|  | DC Power Supply Adapter WXGA (1280*800) |  | SP-5B41, SP-5B41* only |
|  | DC Power Supply Adapter SXGA (1280*1024) |  | SP-5B41, SP-5B41* only |
|  | DC Power Supply Adapter FWXGA (1360*768) |  | SP-5B41, SP-5B41* only |
|  | DC Power Supply Adapter FWXGA (1366*768) |  | SP-5B41, SP-5B41* only |
|  | DC Power Supply Adapter WXGA+ (1440*900) |  | SP-5B41, SP-5B41* only |
|  | DC Power Supply Adapter WXGA++ (1600*900) |  | SP-5B41, SP-5B41* only |
|  | DC Power Supply Adapter FullFD (1920*1080) |  | SP-5B41, SP-5B41* only |
| SP5000 Series (SP-5B90) | SP-5490WA WVGA (800*480) |  |  |
|  | SP-5690WA WXGA (1280*800) |  |  |
|  | SP-5790WA FWXGA (1366*768) |  |  |
| ST6000 Series | ST-6200WA WQVGA（480*272） |  | GP-Pro EX 4.09 SP1 (V4.09.250) or later. Can select the model of ST6000. |
|  | ST-6300TA VGA（640*480） |  |  |
|  | ST-6400WA WVGA（800*480 ） |  |  |
|  | ST-6500TA SVGA（800*600） |  |  |
|  | ST-6500WA WSVGA（1024*600 ） |  |  |
|  | ST-6600WA WXGA（1280*800 ） |  |  |
|  | ST-6700WA FWXGA（1366*768 ） |  |  |
| STM6000 Series | STM-6200WA WQVGA（480*272） |  | GP-Pro EX 4.09 SP1 (V4.09.350) or later. Can select the model of STM6000 Series. |
|  | STM-6400WA WVGA（800*480 ） |  |  |
|  | STM-6B00 WQVGA（480*272） |  |  |
|  | STM-6B00 WVGA（800*480 ） |  |  |

- Use the file for QVGA, “connection_gp4301_q_IAI-ELECYLINDER_ml_V130.prx”.

In the file above, the display unit type is set as shown below;

- Series: GP4000 Series

- Model: GP-4301T

Change models on GP-Pro EX according to a model at transfer destination. For the model change procedure, refer to the attached GP-Pro EX Reference Manual (5.1 Changing a Display Unit).

## Software

Table5-2  Software

| NO | Manufacturer | Product Name | Model | Note |
| --- | --- | --- | --- | --- |
| 1 | Schneider Electric Holdings | GP-Pro EX | PFXEXEDV40 | Ver4.00.000 or later |

GP-Pro EX Ver.4.00.000 or later is required to use the file. If the version of GP-Pro EX you have is earlier than Ver.4.00.000, update the version before use.

To use the file, you need to install IAI ELECYLINDER SIO Driver.

## Connected device

Table5-3  Connected Device

| No | Manufacturer | Product Name | Series | Model | Note |
| --- | --- | --- | --- | --- | --- |
| 1 | IAI Corporation | *1 ELECYLINDER SIO | See Table5-4 | See Table5-4 | *2 ・ELECYLINDER SIO ・File Ver1.30 for ELECYLINDER |

*1　Note that this is not described in the GP-ProEX connection manual.

*2　Use IAI ELECYLINDER SIO with the file.

Table 5-4  Type of ELECYLINDER that can be connected

| Category | Type |  | Model |
| --- | --- | --- | --- |
| Slider | Standard | ー | EC-S3, EC-S4, EC-S6, EC-S7 |
|  |  | Motor return type | EC-S3□R, EC-S4□R, EC-S6□R, EC-S7□R |
|  | Belt-driven | ー | EC-B6, EC-B7 |
|  |  | Bottom-mounted | EC-B6U, EC-B7U |
|  | High rigidity | ー | EC-S6□AH, EC-S7□AH |
|  |  | Motor return type | EC-S6□AHR, EC-S7□AHR |
|  | Large | ー | EC-S13, EC-S13X, EC-S15, EC-S15X |
|  | Ultra-Mini | ー | EC-SL3 |
| Rod | Standard |  | EC-R6, EC-R7 |
|  | Radial Cylinder | ー | EC-RR3, EC-RR4, EC-RR6, EC-RR7 |
|  |  | Motor return type | EC-RR3□R, EC-RR4□R, EC-RR6□R, EC-RR7□R |
|  | High rigidity Radial Cylinder | ー | EC-RR6□AH, EC-RR7□AH |
|  |  | Motor return type | EC-RR6□AHR, EC-RR7□AHR |
|  | Small type |  | EC-RP4, EC-GS4, EC-GD4 |
|  | Dust-proof and drip-proof |  | EC-R6□W, EC-R7□W |
|  | Dust-proof and drip-proof radial cylinder |  | EC-RR6□W, EC-RR7□W |
|  | Ultra-Mini |  | EC-GDS3、EC-GDB3 |
| Table | Small type |  | EC-TC4, EC-TW4 |
|  | Ultra-Mini |  | EC-T3 |
| Stopper cylinder | ー |  | EC-ST15 |
| Rotary | ー |  | EC-RTC9、EC-RTC12 |
|  | Large |  | EC-RTC18 |
| Gripper | ー |  | EC-GRB8、EC-GRB10、EC-GRB13、 |

## How to connect and communication cable diagram

There are two ways to connect to ELECYLINDER, one with using a SIO converter and the other without.

The following shows the connection method (cabling) including the system configuration.

### System Configuration

The settings in the file are for use without using a SIO converter – the communication type is RS485 (on the display unit) -. For use in another configuration, confirm [5.5.2. Cable Diagram] and change the display unit’s connection port and communication settings according to the configuration to be used on GP-Pro EX. For the communication setting change procedure, refer to [12. GP-Pro EX communication setting change].

#### Without using the SIO converter

When the communication type is RS232C (settings on the display unit),

Display Unit

Device/PLC

Figure 5-2

Table 5-5

| Series | Model | Link I/F | Communication type(GP) | Cable diagram | Supplementary |
| --- | --- | --- | --- | --- | --- |
| ELECYLINDER | Connected device | - | RS232C | Cable diagram1 | 1:1 |

When the communication type is RS485 (on the display unit)

Device/PLC

Device/PLC

Device/PLC

Connected devices – 16 units, max.

Display Unit

Figure 5-3

Table 5-6

| Series | Model | Link I/F | Communication type (GP) | Cable Diagram | Supplementary |
| --- | --- | --- | --- | --- | --- |
| ELECYLINDER | Connected device | - | RS-485(2Wire) | Cable diagram2 | 1:1 1:n |

#### When using a SIO converter

Device/PLC

Device/PLC

Device/PLC

SIO

変換器

Connected devices – 16 units, max.

SIO converter

Figure 5-4

Display Unit

Table 5-7

| Series | Model | Link I/F | Communication type (GP) | Cable Diagram | Supplementary |
| --- | --- | --- | --- | --- | --- |
| ELECYLINDER | Connected device | D-sub 9 pin connector on SIO Converter （RCB-TU-SIO-□） | RS-232C | Cable diagram3 | 1:1 1:n |
|  |  | Mini DIN 8 pin connector on SIO Converter （RCB-TU-SIO-□） | RS-232C | Cable diagram4 | 1:1 1:n |

【Note】　・Do not connect a Teaching Box or PC simultaneously with the display unit for use.

（On the SIO converter, the D-sub 9 pin connector and the mini DIN 8 pin connector cannot be simultaneously connected for use.）

### Cable Diagram

The settings in the file are for use in the case of [5.5.2.2. Cable Diagram 2 (RS422/485 2-Wire)].

For use with another cable diagram, change the display unit’s connection port and communication settings according to the configuration to be used on GP-Pro EX. For the communication setting change procedure, refer to [12. GP-Pro EX communication setting change].

#### Cable Diagram 1 (RS232C)

Table 5-7

| Connection Port | Example | Cable and Adapter | - Comment |
| --- | --- | --- | --- |
| GP4000 (COM1)*1 SP5000 (COM1/2)*2 SP5B00 (COM1) | 1A | RS232C conversion unit by IAI Corporation RCB-CV-MW + Communication cable by IAI Corporation CB-RCA-SIO050 | RS232C conversion unit (RCB-CV-MW) and communication cable (CB-RCA-SIO050) are accesories of PC interface Software (RCM-101-MW) by IAI Corporation. |
| LT4000 (COM1) | 1B | RS232C conversion unit by IAI Corporation RCB-CV-MW + RS232C conversion unit by IAI Corporation RCB-CV-MW + Communication cable by IAI Corporation CB-RCA-SIO050 | RS232C conversion unit (RCB-CV-MW) and communication cable (CB-RCA-SIO050) are accesories of PC interface Software (RCM-101-MW) by IAI Corporation. |

*1 Except GP-4203T

*2 Except SP-5B00

1A)

Figure 5-5

1B)

Figure 5-6

#### Cable Diagram 2 (RS422/485 2-Wire)

Table 5-8

| Connection Port | Cable and Adapter |  | - Comment |
| --- | --- | --- | --- |
| GP-4*01TM (COM1) GP-Rear Module (COM1) | 2A | COM port conversion adapter by Pro-face CA3-ADPCOM-01 + Terminal block conversion adapter by Pro-face CA3-ADPTRM-01 + User-created cable 1 + Junction by AMP *1 5-1473574-4 + Controller link cable by IAI Corporation CB-RCB-CTL002 | Cable length: 100m or less |
|  | 2B | User-created cable 1 + Junction by AMP *1 5-1473574-4 + Controller link cable by IAI Corporation CB-RCB-CTL002 |  |
| GP-4*03T *3（COM2） GP-4203T（COM1） | 2C | User-created cable 1 + Junction by AMP *1 5-1473574-4 + Controller link cable by IAI Corporation CB-RCB-CTL002 | Cable length: 100m or less |
| GP4000 *4 (COM2) GP-4201T (COM1) GP4*01TM (COM1) GP-Rear Module (COM1) SP5000 *5 (COM1/2) SP-5B00 (COM2) | 2D | Pro-face RS-422 Terminal Block Conversion Adapter PFXZCBADTM1 *6 + User-created cable 1 + Junction by AMP *1 5-1473574-4 + Controller link cable by IAI Corporation CB-RCB-CTL002 | Cable length: 100m or less |
|  | 2B | User-created cable 1 + Junction by AMP *1 5-1473574-4 + Controller link cable by IAI Corporation CB-RCB-CTL002 |  |
| LT-4*01TM (COM1) LT-Rear Module (COM1) | 2E | RJ45 RS-485 Cable (5m) by Pro-face PFXZLMCBRJR81 + Junction by AMP *1 5-1473574-4 + Controller link cable by IAI Corporation CB-RCB-CTL002 | Cable length: 100m or less |

*1 When using more than one junction by AMP, user-created cable 2 is required.

*2 Only the COM port which can communicate by RS-422/485 (2 wire) can be used.

*3 Except GP-4203T

*4 All GP4000 models except GP-4100 series, GP-4*01TM, GP-Rear Module, GP-4201T and GP-4*03T

*5 Except SP-5B00

*6 When using a Terminal Block Conversion Adapter (CA3-ADPTRM-01) instead of the RS-422 Terminal Block Conversion Adapter, refer to Cable Diagram 2A.

2A)

Figure 5-7

Figure 5-8

2B)

Figure 5-9

Figure 5-10

2C)

Figure 5-11

Figure 5-12

[Important] ・5V output (6pin) of Display must not be connected.

2D)

Figure 5-13

Figure 5-14

Figure 5-15

2E)

Figure 5-16

Figure 5-17

#### Cable Diagram 3 (RS232C)

Table 5-9

| Connection Port | Cable/Adapter |  | - Comment |
| --- | --- | --- | --- |
| GP4000 (COM1)*1 SP5000 (COM1/2)*2 SP-5B00 (COM1) | 3A | User-created cable + SIO converter by IAI Corporation RCB-TU-SIO- + Controller link cable by IAI Corporation CB-RCB-CTL002 | Cable length from Display Unit to SIO converter: 15ｍ or less Cable length from SIO converter to Connected Device: 100ｍ or less |
|  | 3B | User-created cable 1 + SIO converter by IAI Corporation RCB-TU-SIO- + User-created cable 2 + Junction by AMP *3 5-1473574-4 + Controller link cable by IAI Corporation CB-RCB-CTL002 |  |
|  | 3C | User-created cable 1 + SIO converter by IAI Corporation RCB-TU-SIO- + User-created cable 2 + Junction by AMP *3 5-1473574-4 + Controller link cable by IAI Corporation CB-RCB-CTL002 |  |
| LT-4*01TM (COM1) LT-Rear Module (COM1) | 3D | RJ45 RS-232C Cable (5m) by Pro-face PFXZLMCBRJR21 + SIO converter by IAI Corporation RC-TU-SIO- + Controller link cable by IAI Corporation CB-RCB-CTL002 |  |
|  | 3E | RJ45 RS-232C Cable (5m) by Pro-face PFXZLMCBRJR21 + SIO converter by IAI Corporation RC-TU-SIO- + User-created cable 1 + Junction by AMP *3 5-1473574-4 + Controller link cable by IAI Corporation CB-RCB-CTL002 |  |
|  | 3F | RJ45 RS-232C Cable (5m) by Pro-face PFXZLMCBRJR21 + SIO converter by IAI Corporation RC-TU-SIO- + User-created cable 1 + Junction by AMP *3 5-1473574-4 + Controller link cable by IAI Corporation CB-RCB-CTL002 |  |

*1 Except GP-4203T.

*2 Except SP-5B00.

*3 When using more than one junction by AMP, user-created cable 3 is required.

3A)

Figure 5-18

Figure 5-19

3B)

Figure 5-20

Figure 5-21

3C)

Figure 5-22

Figure 5-23

3D)

Figure 5-24

Figure 5-25

3E)

Figure 5-26

Figure 5-27

3F)

Figure 5-28

Figure 5-29

#### Cable Diagram 4 (RS232C)

Table 5-10

| Connection Port | Cable/Adapter |  | - Comment |
| --- | --- | --- | --- |
| GP4000 (COM1)*1 SP5000 (COM1/2)*2 SP-5B00 (COM1) | 4A | RS232C conversion unit by IAI Corporation RCB-CV-MW + Communication cable by IAI Corporation CB-RCA-SIO050 + SIO converter by IAI Corporation RCB-TU-SIO-□ + Controller link cable by IAI Corporation CB-RCB-CTL002 | Cable length from SIO Converter to Connected Device: 100ｍ or less |
|  | 4B | RS232C conversion unit by IAI Corporation RCB-CV-MW + Communication cable by IAI Corporation CB-RCA-SIO050 + SIO converter by IAI Corporation RCB-TU-SIO-□ + User-created cable 1 + Junction by AMP *3 5-1473574-4 + Controller link cable by IAI Corporation CB-RCB-CTL002 |  |
|  | 4C | RS232C conversion unit by IAI Corporation RCB-CV-MW + Communication cable by IAI Corporation CB-RCA-SIO050 + SIO converter by IAI Corporation RCB-TU-SIO-□ + User-created cable 1 + Junction by AMP *3 5-1473574-4 + Controller link cable by IAI Corporation CB-RCB-CTL002 |  |

*1 Except GP-4203T

*2 Except SP-5B00

*3 When using more than one junction by AMP, user-created cable 2 is required.

【NOTE】　　RS232C Conversion Unit (RCB-CV-MW) and Communication Cable (CB-RCA-SIO050) are accessories of PC-compatible Software by IAI Corporation (RCM-101-MW).

4A)

Figure 5-30

Figure 5-31

4B)

Figure 5-32

Figure 5-33

4C)

Figure 5-34

Figure 5-35

## Communciation Settings

### GP-Pro EX Communication Settings

Change the settings according to the communication settings of the ELECYLINDER unit and the connection configuration to be used. For the communicaton setting change procedure, refer to [12. GP-Pro EX communication setting change]. For the default of the communication settings of the ELECYLINDER unit, refer to the table 5-12.

For the file, 16 that is the maximum number of connected devices is selected because of the feature that checks connection axes.

Even if the number of connected ELECYLINDER units is less than 16, be sure to select 16 for the number of connected devices in the communication settings.

[NOTE] If the Allowable Number of Devices/PLCs of the connected devices is deleted, operation is affected. Even when the number of connected devices is small, keep 16 in the setting for use.

Figure 5-36  GP-Pro EX Communication Settings

Table 5-11  Communication Settings

| Item | Default | Range |
| --- | --- | --- |
| Communication type | RS-422/485(2-wire) | RS232C / RS-422/485(2-wire) |
| Baud rate | 38400 (bps) | 2400 / 4800 / 9600 / 19200 / 38400 57600 / 115200 |
| Date length | 8 bits | Fixed |
| Parity | None | Fixed |
| Stop bit | 1 bit | Fixed |
| Flow control | None | Fixed |
| Timeout | 3 (sec) | 1 to 127 |
| Retry | 2 | 0 to 255 |
| Wait to send | 2 (ms) | 0 to 255 |
| Text data mode | 1 | 1 to 8 |

[NOTE] The communication type setting differs depending on a connection method. For RS232C connection, change the setting before use.

### Communication Settings of Devices/PLCs

- The default values of the ELECYLINDER communication settings are as shown below.

Table 5-12　Device/PLC communication settings (Default)

| Communication settings | On Device/PLC |
| --- | --- |
| Baud rate | 38400 (bps) |
| Data length | 8 bits |
| Parity | None |
| Stop bit | 1 bit |

### Indirect Device Settings

The file has used Indirect Device Settings. If a device ID address of Indirect Device Settings is changed, operation will be affected. Be cautious when revising the Device/PLC settings.

Refer to Chapter 7.5 in GP-Pro EX Reference Manual.

Table 5-13　Device/PLC communication settings (Default)

| Indirect Device | Address |
| --- | --- |
| Indirect1 | [#INTERNAL]USR29980 |

### Device-Specific Settings

For the file data, 16 is selected for the allowable number of Devices/PLCs.

Configure settings as shown in the table 5-8 below for the 16 units. If the settings are different, operation will be affected.

Table 5-14　Device-Specific Settings

| No. | Device Name | Settings | Device ID |
| --- | --- | --- | --- |
| 1 | EC00 | Axis No.=0 | 1 |
| 2 | EC01 | Axis No.=1 | 2 |
| 3 | EC02 | Axis No.=2 | 3 |
| 4 | EC03 | Axis No.=3 | 4 |
| 5 | EC04 | Axis No.=4 | 5 |
| 6 | EC05 | Axis No.=5 | 6 |
| 7 | EC06 | Axis No.=6 | 7 |
| 8 | EC07 | Axis No.=7 | 8 |
| 9 | EC08 | Axis No.=8 | 9 |
| 10 | EC09 | Axis No.=9 | 10 |
| 11 | EC10 | Axis No.=10 | 11 |
| 12 | EC11 | Axis No.=11 | 12 |
| 13 | EC12 | Axis No.=12 | 13 |
| 14 | EC13 | Axis No.=13 | 14 |
| 15 | EC14 | Axis No.=14 | 15 |
| 16 | EC15 | Axis No.=15 | 16 |

# Screen Configuration

## Screen Type

The file provides the following 7 kinds of feature screens.
The screen number, B9990, B9992, B9993, B9994, B9995, B9996, B9997, and W2000 cannot be edited. And the screen numbers cannot be changed.

Table 6-15  Screen Type

| Screen Title | Screen Image | Feature |
| --- | --- | --- |
| Axis Select Screen Number B9990 |  | - Check connection status of 16 axes - Select connection axes. |
| Axis No./Name Screen Number B9992 |  | - Change the specified axis number. - Change the name of the specified axis number. |
| Simple Data Setting Forward Condition Screen Number B9993 |  | - Specify ourtward. - Transfer after settings - Switch units |
| Simple Data Setting Backward Condition Screen Number B9994 |  | - Specify return. - Transfer after settings - Switch units. |
| Simple Data Setting Position Setting Screen Number B9995 |  | - Specify position data. - Transfer after settings |
| Simple Data Setting Manual Mode Screen Number B9996 |  | - Manual operation |
| ELECYLINDER Type Image (ELECYLINDER image screen) Screen Number B9997 |  | - B9995 uses the screen. |
| TransferConfirmationScreen Screen Number W2000 |  | - Window Screen for confirmation of data transfer |

## Screen Transition

Start

Home　Switch

【B1】Home Screen

【B9993】Simple Data Setting

【B9992】Axis No./Name

Home

Switch

【B9990】Axis Select

【B9994】Simple Data Setting - Backward

【B9995】Simple Data Setting - Forward

【B9996】Simple Data Setting–Manual Mode

Figure 6-37  Screen Transition

# Details of Screens

## Home Screen (B0001: Edit allowed）

### Overview

This is the Home screen displayed at the time of starting the display unit. The Home screen can be edited. To separately create a Home screen, you can delete this screen and replace it with another screen.

### Screen Image

2

1

Figure 7-38  Home Screen

Table 7-16  Descriptions of parts

| No. | Item | Parts | Descriptions |
| --- | --- | --- | --- |
| 1 | Axis Select | Switch | Switches to the Axis Select screen (B9990). To separately create a Home screen, place a switch that switches the screen to the screen number, ”9990”. |
| 2 | Home Screen Number Specification | Data Display | Screen number of Home. The screen with the number specified here is operated as a Home screen. When the Home switch of the header is pressed, the screen is changed to the one with the specified number. (“1” in the file). Or when a value is entered in [#INTERNAL]USR29999, the screen number of the entered value means the Home screen. |

## Axis Select (B9990: Edit not allowed)

### Overview

This screen checks connection status with the ELECYLINDERs (Axis 1 to 16) and selects an axis number of setting target. To check connection status of the ELECYLINDER, touch the [Reconnection] switch. After checking the connection status, selecting an axis allows you to check or change values of the selected axis.

### Screen Image

1

2

5

4

３

6

Figure 7-39  Axis Select Screen

Table 7-17  Descriptions of parts

| No. | Item | Parts | Descriptions |
| --- | --- | --- | --- |
| 1 | Home | Switch | Switches to the Home screen. The value specified for [#INTERNAL]USR29999 becomes the Home screen number. |
| 2 | Simple Data Setting | Switch | Switches to the Simple Data Setting screen. |
| 3 | Axis No./Name | Switch | Switches to the Axis No./Name screen. |
| 4 | Reconnection | Switch | Starts checking connection status of Axis 1 to 16. |
| 5 | Axis Select | Switch | The axis names of ELECYLINDERs with connection confirmed are shown and can be selected. When the axis name of ELECYLINDER at connection destination is empty, the text, [EC] is shown. When no ELECYLINDER is connected or any ELECYLINDER out of support target is connected, you cannot select axes because of grayout display. |
| 6 | Axis Select Screen Change | Switch | The screen showing axis 00 to 07 can be changed to the one showing axis 08 to 15. The screen showing axis 08 to 15 can be changed to the one showing axis 00 to 07. |

## Axis No./Name (B9992: Edit not allowed)

### Overview

You can change axis numbers and names of ELECYLINDERs. Press the [Execute] switch after entering them to change them. When axis numbers and axis names are changed, be sure to run [Reconnection] on the Axis Select screen.

### Screen Image

1

2

3

4

5

Figure 7-40  Axis No. /Name Screen

Table 7-18  Descriptions of parts

| No. | Item | Parts | Descriptions |
| --- | --- | --- | --- |
| 1 | Return | Switch | Switches to 1-upper level (Axis Select) |
| 2 | Home | Switch | Switches to the Home screen. The value specified for [#INTERNAL]USR29999 becomes the Home screen number. |
| 3 | Axis Number | Data Display | Specify an axis number. Change the selected axis to a different axis number. |
| 4 | Axis Name | Data Display | Specify an axis name. Up to 12 single-byte characters can be entered. |
| 5 | Execute | Switch | Writes the axis number and the axis name to the ELECYLINDER selected on the Axis Select screen. |

## Forward Condition Screen (B9993: Edit not allowed)

### Overview

You can set forward operating conditions of the selected axis on this screen.

The data is received from the selected axis when this screen is displayed. Even if you change each setup value, it will not be reflected to the ELECYLINDER - writing process -. To write each setup value to the ELECYLINDER, touch the [Transfer] switch after changing the values and run writing process to the ELECYLINDER. After the data is written, the axis data is automatically received again and the latest data is shown.

### Screen Image

1

2

9

6

4

5

3

7

10

11

8

12

Figure 7-41  Simple Data Setting Screen

Table 7-19  Descriptions of parts

| No. | Item | Parts | Descriptions |
| --- | --- | --- | --- |
| 1 | Return | Switch | Switches to 1-upper level (Axis select). |
| 2 | Home | Switch | Switches to the Home screen. The value specified for [#INTERNAL]USR29999 becomes the Home screen number. |
| 3 | Acceleration | Data Display (Input allowed） | Specify acceleration. ・Input value (unit: ％） ・Input value (unit: G) Possible to input with every unit above by unit change. |
| 4 | Velocity | Data Display （Input allowed） | Specify velocity. ・Input value (unit:%) ・Input value (unit:mm/s) Possible to input with every unit above by unit change. |
| 5 | Deceleration | Data Display （Input allowed） | Specify deceleration. ・Input value (unit:％) ・Input value (unit:G) Possible to input with every unit above by unit change. |
| 6 | Push | Switch | With or without use of push for forward （OFF：Disabled, ON: Enabled） Enable it to show the setting items of [Push Force] and [Push st. p. -Push start point-]. |
| 7 | Push Force | Data Display (Input allowed) | Specify ‘Push Force’. ・Input value (unit:%) ・Input value (unit:N) Possible to input with every unit above by unit change. The push force can be specified only when the box of [Push] is checked. |
| 8 | Switch to Backward Condition Screen | Switch | Switches to Backward Condition Screen. |
| 9 | Switch to Position Setting Screen | Switch | Switches to Position Setting Screen. |
| 10 | Switch to Manual Mode Screen | Switch | Switches to Manual Mode Screen. |
| 11 | Unit Change | Switch | Switches 2 kinds of units. Switching units can switch the values of forward and backward of operating conditions. ・Unit ％ ・Unit mm/s, G |
| 12 | Transfer | Switch | Writes the setup data to the ELECYLINDER. |

## Backward Condition Screen (B9994: Edit not allowed)

### Overview

You can set backward operating conditions of the selected axis on this screen.

When the Simple Data Setting Screen - Forward Condition Screen - is displayed, the data is received from the selected axis. Even if you change each setup value, it will not be reflected to the ELECYLINDER - writing process -. To write each setup value to the ELECYLINDER, touch the [Transfer] switch after changing the values and run writing process to the ELECYLINDER. After the data is written, the axis data is automatically received again and the latest data is shown.

### Screen Image

1

2

9

6

4

5

3

7

10

11

8

12

Figure 7-42  Simple Data Setting Screen

Table 7-20  Descriptions of parts

| No. | Item | Parts | Descriptions |
| --- | --- | --- | --- |
| 1 | Return | Switch | Switches to 1-upper level (Axis select) |
| 2 | Home | Switch | Switches to the Home screen. The value specified for [#INTERNAL]USR29999 becomes the Home screen number. |
| 3 | Acceleration | Data Display (Input allowed） | Specify acceleration. ・Input value (unit: ％） ・Input value (unit: G) Possible to input with every unit above by unit change. |
| 4 | Velocity | Data Display （Input allowed） | Specify velocity. ・Input value (unit:%) ・Input value (unit:mm/s) Possible to input with every unit above by unit change. |
| 5 | Deceleration | Data Display （Input allowed） | Specify deceleration. ・Input value (unit:％) ・Input value (unit:G) Possible to input with every unit above by unit change. |
| 6 | Push | Switch | With or without use of push for backward （OFF：Disabled, ON: Enabled） Enable it to show the setting items of [Push Force] and [Push st. p. -Push start point-]. |
| 7 | Push Force | Data Display (Input allowed) | Specify ‘Push force’. ・Input value (unit:%) ・Input value (unit:N) Possible to input with every unit above by unit change. The push force can be specified only when the box of [Push] is checked. |
| 8 | Switch to Forward Condition Screen | Switch | Switches to Forward Condition Screen. |
| 9 | Switch to Position Setting Screen | Switch | Switches to Position Setting Screen. |
| 10 | Switch to Manual Mode Screen | Switch | Switches to Manual Mode Screen. |
| 11 | Unit Change | Switch | Switches 2 kinds of units. Switching units can switch the values of forward and backward of operation condition. ・Unit ％ ・Unit mm/s, G |
| 12 | Transfer | Switch | Writes the setup data to the ELECYLINDER. |

## Position Setting Screen (B9995: Edit not allowed)

### Overview

You can configure position settings of the selected axis on this screen.

When the Simple Data Setting-Forward Condition Setting- is displayed, data is received from the selected axis. Even if you change each setup value, it will not be reflected to the ELECYLINDER - writing process -. To write each setup value to the ELECYLINDER, touch the [Transfer] switch after changing the values and run writing process to the ELECYLINDER. After the data is written, the axis data is automatically received again and the latest data is shown.

### Screen Image

1

2

7

5

3

4

8

6

9

Figure 7-43  Simple Data Setting Screen

Table 7-21  Descriptions of parts

| No. | Item | Parts | Descriptions |
| --- | --- | --- | --- |
| 1 | Return | Switch | Switches to 1-upper level (Axis select) |
| 2 | Home | Switch | Switches to the Home screen. The value specified for [#INTERNAL]USR29999 becomes the Home screen number. |
| 3 | B.End | Data Display (Input allowed under a condition) | Configure position settings of backward end. ・Input value (unit:ｍｍ) The location of B.End differs depending on the position of the home and the state of push. Changing units affects nothing. |
| 4 | F.End | Data Display (Input allowed under a condition) | Configure position settings of forward end. ・Input value (unit:ｍｍ) The location of F.End differs depending on the position of the home and the state of push. |
| 5 | Push start point (Push st. p.) | Data Display (Input allowed) | Specify the start position of Push. ・Input value (unit:ｍｍ) |
| 6 | Switch to Forward Condition Screen | Switch | Switches to Forward Condition Screen. |
| 7 | Switch to Backward Condition Screen | Switch | Switches to Backward Condition Screen. |
| 8 | Switch to Manual Mode Screen | Switch | Switches to Manual Mode Screen. |
| 9 | Transfer | Switch | Writes the setup data to the ELECYLINDER. |

#### Position Setting

The display images differ depending on a model of ELECYLINDER to be connected. Three kinds of models can be displayed.

・Slider

Figure 7-44  Slider

・Rod

Figure 7-45  Rod

・Table

Figure 7-46  Table

・Rotary

Figure 7-47  Rotary

・Gripper

Figure 7-48  Gripper

## Simple Data Setting (B9996: Edit not allowed)

### Overview

Manual operation of the selected axis is possible on this screen.

This screen usually shows the latest state of axis data. If the setup value is changed, it will be written to the ELECYLINDER.

### Screen Image

1

2

8

5

3

4

9

7

6

Figure 7-49  Simple Data Setting Screen

Table 7-22  Descriptions of parts

| No. | Item | Parts | Descriptions |
| --- | --- | --- | --- |
| 1 | Return | Switch | Switches to 1-upper level (Axis select) |
| 2 | Home | Switch | Switches to the Home screen. The value specified for [#INTERNAL]USR29999 becomes the Home screen number. |
| 3 | Operation change | Switch | Radio switches that selects Test run, Jog, or Inching. The operation of the move switch differs depending on this setting. |
| 4 | Servo ON/Brake | Swtich Lamp | Switches the servo’s power ON and OFF and the brake’s release ON and OFF. |
| 5 | Current Position | Data Display (Input not allowed) | Shows the current position data. |
| 6 | Move switch | Switch | Push F.End to move to the F.End side. Push B.End to move to the B.End side. The operation differs depending on a setting state. |
| 7 | Switch to Forward Condition Screen | Switch | Switches to Forward Condition Screen. |
| 8 | Switch to Backward Condition Screen | Switch | Switches to Backward Condition Screen. |
| 9 | Switch to Position Setting Screen | Switch | Switches to Position Setting Screen. |

#### Manual Mode

For the manual mode switches, display of switches differs depending on a state of ‘Homing’ comleted or not completed.

・For home direction standard and homing not completed

Figure 7-50  Homing switch (home standard)

・For home direction reverse and homing not completed,

Figure 7-51  Homing switch (home reverse)

・For homing completed,

Figure 7-52  After homing is completed

## Transfer Confirmation Screen (W2000: Edit not allowed）

### Overview

The confirming message window appears before the data specified on the Simple Data Setting screen is written to the ELECYLINDER. Push [Yes] to start writing the data to the ELECYLINDER. Push [No] to stop writing and return to the original screen.

### Screen Image

1

2

Figure 7-53  Transfer Confirmation Screen

Table 7-23  Descriptions of parts

| No. | Item | Parts | Descriptions |
| --- | --- | --- | --- |
| 1 | Yes | Switch | Writes data to ELECYLINDER. Receives the data after writing is completed and updates to the latest data. |
| 2 | No | Switch | Closes the dialog box without writing. |

# Global D Script

The file does not use Global D Script.

# Address Map

## List of used internal addresses

Table 9-24  Address Map

| Address | Form | Details |
| --- | --- | --- |
| USR29599 | 16bit | For languages change, change control address You can change this address. No problem. |
| USR29600 to USR29998 | 16bit | ELECYLINDER parts reserve area [Important] This address area must not be used. If you edit it, for example, replace addresses, the parts in the file do not properly behave. Make sure that the addresses in this range are not included at the time of edit such as block address conversion. |
| USR29999 | 16bit | Home screen number Input the screen number of the Home screen. Switch to the screen number that is input in this address at the time of touching the [Home] switch. |

## Variable List

The file does not use Global D Script.

# Text Table

## Table1（Table 1）

Table 10-25  Address Map

| NO, | Text | Details |
| --- | --- | --- |
| 9900 to 10000 | Japanese Chinese English | ELECYLINDER parts reserve area. This area is being used for the ELECYLINDER parts. If the texts in this area are changed, they are not properly displayed. |
| Other |  | Settings To use the language change feature – Text Table -, it’s necessary to specify change control addresse. You can change this address according to project data. In this file, it’s [#INTERNAL]USR29599. This address can be changed. |

# Incorporating project data

## To create a project data file based on the file

When you create a project data file based on the file, pay attention to the following items.

・Duplicate adddresses

・Duplicate text tables

・Duplication of base screen and window screen

## When incorporating the file in the created project file

When incorporating the file in the already created project file, follow the steps below.

If the steps are not followed, it cannot be properly incorporated.

- Device/PLC Setting (Settings of 16 device units, Registration of indirect devices)
Be sure to add up to 16 connected devices with [Add Device/PLC] and configure indirect device name settings before copying the screen. For the setting procedure, refer to 11.2.1. Device/PLC Settings.

- Text Table (No,9900 to No,10000)
・　Export the text table in the file.

(Example;　StringTable.txt　　Can be saved as any filename.)
・　Import the exported file to the project file at destination.
・　The texts registered in the file are Japanese, Chinese, and English.
・　When loading, load them to the table of each language.
・　When the text table is not used, load them to the table 1, 2, and 3.

- Copy From Another Project
Select the file on [Copy From Another Project] and load the Base Screen and the Window Screen. When loading the window screen, be sure to select 2000 for Copy-To Start Screen Number before copy.

### Device/PLC Settings

Configure settings according to the contents of 5.6 Communication Settings.

To newly add a connected device, configure settings of the contents in 5.4 Connected device and add a connected device when the dialog box of Figure 11-1 appears.

Figure 11-54  Add Device/PLC

Configure Device-Specific settings of the added devices.

Add up to 16 (max.) devices and specify axis numbers, and the state will be as shown in the figure 11-2 below. For details of the settings, refer to 5.6.1Device-Specific Settings.

Figure 11-55  Add 16 devices

Next, click on the icon of Add Indirect Device of No. 1, and the state will be as shown in the

figure 11-3 below.

When the state is as shown in the figure, the settings of the connected device are completed.

Figure 11-56  Add indirect devices

### Import and export of text table

Export the text table of the file.

The table registered in the file is [1] [2][3]. Select [1] [2][3] in the export table and export it.

Figure 11-57  Export Text Table

Next, import the exported file to the project file you use.
Before importing it, make sure that the text No. described in the chapter ‘10. Text Table’ is blank in the text table that is an import destination. If text is input, it will be overwritten at the time of import. If you don’t change the import destination, it will be imported to the text table number [1][2][3].

Figure 11-58  Import Text Table

### Copy From Another Project

For Copy From Another Project, no screen is necessary except 2 kinds of screens.

Disable all except Base Screen and Window Screen.

・Base Screen　B9990、B9992、B9993、B9994、B9995、B9996、B9997

・Window Screen　W2000

Figure 11-59  Copy From Another Project

When copy is properly completed, Figure 11-7 appears.

Figure 11-60  Comletion message of Copy From Another Project

Confirm the copy result with the Screen List. If there are B0001, B9990, B9992, B9993, B9994, B9995, B9996, B9997, and W2000 in the Screen List, copy is completed. (Figure 11-8)

The screens in red cannot be edited and the details of screen settings cannot be confirmed.

Figure 11-61  Screen List when copy is properly completed

# Incorporating project data

## Change the communication port.

On the GP-Pro EX menu bar, select [Project]->[System Settings]->[Device/PLC].

And then, select [Change Device/PLC].

Change the [Port] to COM1 or COM2 according to the connection configuration to be used and push the [Change] button to confirm the setting.

## Communication Setting Change (the communication type included)

On the GP-Pro EX menu bar, select [Project]->[System Settings]->[Device/PLC].

Set the type described in the cable diagram to be used for the communication type.

Each item of Speed, Data Length, Parity, and Stop Bit must be the same as that of the ELECYLINDER to be connected. And close the [Device/PLC] setting screen. The communication type that can be used differs depending on a display unit type or communication port. When changing the communication type, select a display unit type and a communication port at first according to the connection configuration to be used.